Network Topology - part 1

Definition: Arrangement of the nodes of a computer network.

Topology = Layout

- Physical Topology - Placement of various nodes – in physical world

- Logical topology - Deals with the data flow in the network

Network topology:

- Bus

  - All data transmitted between nodes in the network is transmitted over this common transmission medium and is able to be received by all nodes in the network simultaneously.

  - A signal containing the address of the intended receiving machine travels from a source machine in both directions to all machines connected to the bus until it finds the intended recipient.

- Ring

  - A ring topology is a bus topology in a close loop.

  - Peer to peer LAN topology.

  - Two connections: one to each of its nearest neighbors

  - Unidirectional

  - Sending and receiving data takes place with the help of a token.

  - Whoever has the token, it means it is their turn to send the data.

- Star

  - Every node is connected to a central node called a hub or switch

  - Centralized Management

  - All traffic must pass through the hub or switch.

- Mesh

  - Each node is directly connected to every other nodes in the network.

  - Fault tolerant and reliable.

- Hybrid

  - One or more different types of topologies.